load_hpgl Documentation

One of the graphics window types in the Integral HP-UX OS Release 5.0

is a plotter emulator window that interprets HP-GL commands.  The basic

features of HP-GL commands are implemented on most Hewlett-Packard

Plotters.  Applications programs that use features can attain a high

degree of device independence in sending commands to the display or to

a plotter.  The HP-GL reference manual for the Integral is published as

part of the Programmer's Documentation, (HP part #82865J).  Note that

the Programmer's Documentation is also shipped with the C Compiler,

(Release 5.0.1 or later) for the Integral.

Note also that a fully functional HP-GL window type is included in the

Operating System ROM for the Integral.  Thus it is not necessary to run

load_hpgl in order to access this functionality.  The load_hpgl command

corrects a minor defect in the implementation of the HP-GL plotter

emulator.  Specifically a table in the OS ROM is replaced with a table

in RAM when load_hpgl is run.  A description of the defect corrected

follows.

Approximately 4K bytes of memory are consumed by running load_hpgl.

These 4K bytes are used to store table information used by the plotter

emulator window type.  These bytes are not returned to the user during

the normal operation of the Integral.  The table is lost, as are all

contents of RAM, when the system is powered down.   If load_hpgl is

needed for proper system functionality,  then it should be run by an

Autost program at power up.  Note that load_hpgl should be run only

once after power on time.  Each time load_hpgl is run it will use

another 4K bytes of memory.

The defect in the HP-GL plotter emulator occurs when using the

plot relative (PR) command with negative fractional 'x' values.

The parser incorrectly considers the fractional part of the

parameter as a positive value and adds it to the negative

integer part of the parameter, thus a value of -1.4 becomes

-1 + 0.4 or -0.6.

In order to demonstrate the problem and to verify the correction in the

system, you can send the following HP-GL commands to a graphics window

on the Integral.  If the defect has been corrected then this command

sequence will draw a rectangle in the graphics window.

pg;

sc-1,1,-1,1;

pu;

pu0,0;

pa-.25,-.25;

pd;

pr0,.5;

pr.5,0;

pr0,-.5;

pr-.5,0;

pu;

Related files needed for the load_hpgl program are:

dload

hp-ux.5.0.0

hpgl.data