Plotter_is Revision 1.8

GVG / 31 December 1986

The latest revision of the "plotter_is" utility supports HP-IB and serial

plotters, as well as the creation of plotter windows on both the IPC's internal

EL panel and external monitors; furthermore, the size and location of the

plotter window can be specified.

"Plotter_is" is used to select the current plotter as follows:

plotter_is graphics-window-name [optional-parameters ]

plotter_is HP-IB-address

plotter_is RS-232-device-name [optional-parameters ]

Parameters that may optionally follow a graphics window name are:

x_origin   y_origin   width   height

This allows specifying the location and size of the window.  For example, to

create an HP-GL window named "scratch" that fills the entire EL panel,

"plotter_is" is invoked as:

plotter_is scratch 0 0 512 255

Note that if the parameters are used, ALL FOUR must be specified.

"Plotter_is" can be used to create HP-GL windows on external displays (when

the IPC is used with a SPECTRON video card).  Though this is not hard to do,

there are some complications in creating HP-GL windows on external displays

that a user must be aware of.

The SPECTRON video card is controlled by a software utility called

"scan_video".

If "scan_video" is invoked without parameters, the external monitor becomes the

"primary' display.  Windows on the external display are then found in a

directory called "/dev/screen", while windows on the internal display are found

in a directory called "/dev/intscreen".

Windows can be created on the external display, just as before; for example, to

create an HP-GL window named "scratch" that fills the entire monitor display,

"plotter_is" is invoked as:

plotter_is scratch 0 0 640 400

HOWEVER - a user will NOT be able to access this display through the

"/dev/plotter" device file; the user must access it through the HP-GL window's

full pathname, which, in this case, is "/dev/screen/scratch".

(The problem with "/dev/plotter" is an operating-system bug.  There is no way

under ANY conditions to access an HP-GL window on an external display through

"/dev/plotter", unless a user uses the "ln" utility to link the "/dev/plotter"

name to the HP-GL window name - which is more bother than it's worth,

considering that the window can be accessed under its own pathname anyway.)

An HP-GL window can be created on the internal display by specifying the full

pathname; for example, to create an HP-GL window, named "scratch", on the

internal display, "plotter_is" is invoked as:

plotter_is /dev/intscreen/scratch 0 0 512 255

THIS window can then be accessed using the "/dev/plotter" device file.

This sounds straightforward enough - unless a user invokes "scan_video" with

the "-i" option.  Windows on the internal display are then found in a directory

called "/dev/screen", while windows on the external display are found in a

directory called "/dev/extscreen1".

Windows can be created on the internal display, just as they are if no external

display is present:

plotter_is scratch 0 0 512 255

An HP-GL window can be created on the external display by specifying the full

pathname; for example, to create an HP-GL window, named "scratch", on the

external display, "plotter_is" is invoked as:

plotter_is /dev/extscreen1/scratch 0 0 640 400

In theory, there can be up to seven external displays attached to the IPC.

Should such an unlikely circumstance come to pass, HP-GL windows can be created

on the external displays using the appropriate pathnames:

/dev/extscreen1/...

/dev/extscreen2/...

...

/dev/extscreen7/...

Parameters for a serial plotter are as follows:

baud rate:  110   300     600     1200    2400    4800    9600

parity:     odd   even    no-parity

pacing:     xon-xoff      hardware-handshake      no-pacing

The default conditions are 9600 baud, no parity checking and no pacing.  For an

example, to configure a serial 7475 plotter that is set to 9600 baud and no

parity, "plotter_is" is invoked as:

plotter_is cul00 9600 no-parity hardware-handshake