print_screen

If you run "print_screen" without any parameters, the image on the screen will

be dumped to the default printer - almost exactly what happens when you press

SHIFT/Print (except THAT always goes to the ThinkJet!)

However, "print_screen" has a bewildering array of options that allow it to be

used as a very general-purpose utility for handling screen graphics.

All the options for this utility are:

window <window_name>            Print a window, rather than the entire

screen.

laserjet                        Set options for a LaserJet.

expanded                        Double the size of the print.

sideways                        Print the image sideways.

centered                        Print the image centered vertically.

inverted                        Invert the image.

form_feed                       Do a form-feed after printing the image.

narrow                          Make the print half-width on a ThinkJet

printer.

density <number_of_dots>        Specify dots-per-inch for a LaserJet.

width <number of bytes>         Specify width of paper in bytes

(default is 80).

height <dot_rows>               Specify height of paper in dot rows

(default is 7040).

indent <number_of_bytes>        Specify indentation of image from left

margin (centering is default).

pad_top <dot_rows>              Number of blank dot rows before image.

pad_bottom <dot_rows>           Number of blank dot rows after print.

lines_top <number_of_lines>     Number of blank character lines before print.

lines_bottom <number_of_lines>  Number of blank character lines after print.

part <x y w h>                  Print a portion of the screen where the

origin (0,0) is at the upper-left-hand

corner of the screen, "x" & "y" are the

coordinates of the upper-left-hand corner of

the desired region (in dots, relative to the

origin), and "w" & "h" are the width and

height, respectively, of the desired region

(in dots).

frame                           Print a frame (default).

unframed                        Don't print a frame.

output <file_name>              Send the image to a file instead of a

printer.

adjust                          Adjust the print to have a height that is a

multiple of 15 dots, so it corresponds to an

integral number of printed rows on the

ThinkJet.

You can mix and match options as needed - as long they don't directly

contradict each other; with a little experimentation, you should be able to

use "print_screen" to generate strip charts, banners, and other unusual

printouts.

NOTE: Using GraphicArt With Print_Screen:

The "print_screen" utility is particularly useful for printing images produced

by the IPC's "GraphicArt" drawing program.

However, there are a few details you need to know to get these two programs

to work together.

First, in getting "print_screen" to print the image in the "GraphicArt"

window you need to be aware of an embedded blank in the GraphicArt window name.

At first glance you may try to mistakenly type:

print_screen window GraphicArt

Sadly, this does not work.

The trick here is "GraphicArt" actually uses TWO windows - a "help" window

named "GraphicArt", and a graphics window named "GraphicArt ".

If that last sentence confuses you, take a closer look: the SECOND window name

has a BLANK at the end.  (The programmer wanted to create two windows with the

same name, but that was the only way the operating system would let him do it.)

So - the proper command ACTUALLY is:

print_screen window "GraphicArt "

Notice the blank character at the end of the window name.

Okay, now you can print the window.  The next problem is that, in this

form, the "print_screen" program prints the ENTIRE window, including the "icon

bars" at the sides of the window that provide iconic command menus for the

user.  In most cases, you will NOT want the icon bars printed.

Fortunately, the "print_screen" program allows you to print PART of a

window - by using the "part" option and specifying the coordinate of the

upper-left-hand corner of the desired part and the dimensions (in pixels) of

the part.

The "GraphicArt " window is 490 x 191 pixels in size; each of the icon bars is

40 pixels wide.  The "zero coordinate" of a window is at the

upper-left-hand-corner of the window, so the graphics part of the window

EXCLUDING the icon bars has the coordinate (40, 0), and the dimensions

(410, 191).   That means that the proper command to print the graphics in the

"GraphicArt " window WITHOUT the icon bars is:

print_screen window "GraphicArt " part 40 0 410 191

Finally, you can use the "expanded" option to double the size of the

print, and the "sideways" option to print the expanded image lengthwise on

the page.