LOAD_MODEM (UTIL)               HP Integral PC                LOAD_MODEM (UTIL)

NAME

load_modem - loads the HP 82915A 300/1200 BPS modem driver

SYNOPSIS

load_modem

DESCRIPTION

This command loads the s/w driver module used to control the 300/1200

BPS modem card.  This action is required only for use of the 82915A

plug-in modem card.  Once loaded, the driver is automatically configured

to the appropriate I/O card slot(s) and remains a permanent part of the

operating system until the next time the Integral PC is powered down.

Running this command a second time, after the driver has already been

loaded, has no effect on the system.  Some applications, such as

DATACOMM, automatically invoke this command to simplify user interactions

with the Integral PC.

DIAGNOSTICS

An error will occur if the driver module (modem.data) cannot be found.

["Load failed.  Can not access `modem.data'."]

An error will occur if the OS ROM version is not supported.

["Load failed.  Wrong version."]

An error will occur if there is not sufficient memory to load.

["Load failed.  Out of memory."]

Any other error will generate the following message:

["Load failed."]

A successful load completes with the following message:

["Ready for use."]

FILES

load_modem               dynamic modem driver load utility.

modem.data               modem driver object code module.